|  | ---INTRODUCTION--- |
| --- | --- |
| 1 | Slide only |
| 2 | Slide #1 with spin in of “WHAT YOU NEED TO KNOW” Hi, I’m _______________ I think that we can all agree that we don’t need any more problems. Many of our problems today, are related to actual or potential, “Large Scale Disasters”. It wasn’t that long ago that when asked the question: What is your biggest safety fear for you and your family? The answer would often be: |
| 3 | “The fear of a Terrorist attack! In previous programs, as I would try to promote a concern over House Fires, I would compare the numbers of deaths resulting from Terrorist attacks, both foreign and domestic, to the numbers of civilian fire fatalities. I would show that it would take an event like the 9/11 attack on the World Trade Center every year just to equal the annual numbers of fire deaths in the US. Our greater concern of danger, due to terrorist attack, as opposed to concern over danger from a common house fire was likely due to a couple of reasons: Terrorist incidents (foreign and domestic) are relatively new, to us anyway, and unknown, and unpredictable. Second… Fire Safety we learned as kids, we know you don’t play with fire, we learned Stop, Drop, and Roll, we learned that if our smoke detector alarms, we roll out of bed, stay low, feel the back of the door, checking for heat, we crawl under smoke and we meet at our decided upon meeting place outside. So, since we are very familiar with the precautions, safety measures, and procedures involving house fires, we are usually not that concerned with it. (In other words, …we feel that this problem of house fires, has been taken care of). |
| 4 | Well….This previous fear or concern that we may have had of a terrorist attack, has likely changed with the emergence of and on-going threat of the Coronavirus. ….and to further complicate matters…. Recently we are dealing with: |
| 5 | multiple hurricanes, |
| 6 | blizzards, |
| 7 | forest fires, in addition to a pandemic…. and let’s not forget: |
| 8 | tornados, |
| 9 | and the reoccurring annual flu season. Some of these disasters affect all of us, some may only affect us depending on our location and situation. What makes these disasters even worse, is that, we as individuals, in most cases, can’t do much to avoid these situations, all we can do is follow the recommendations of the experts for prevention and/or recovery after they have already occurred. At this point most of us are thinking that it can’t get any worse….so… Why am I wasting your time with a problem like: |
| 10 | House Fires? Having a variety of current problems or concerns, doesn’t mean that we can afford to minimize or overlook previous problems or concerns. What makes this even more alarming, and in some cases deadly, is that some of these “problems” are not that well known, or may be new. Information involving consequences or “side effects” of new technology is not always readily available to the public. Research also may have uncovered a problem with a long accepted safety measure….or…the safety measure that we had faith in, just may not work or may not work in certain situations. ……... But on a more positive note: often through awareness, we can avoid these more personnel or “Small-Scale” Disasters. Unlike “Large-Scale” Disasters, in which avoidance is probably out of our control, “Small –Scale Disasters” (in this case home fire fatalities) may be avoidable. This program has been designed just for the purpose of preventing us from being blindsided by a “Small-Scale” disaster while our attention is diverted in other directions. Fire deaths, are something that we can do something about. We would hate to survive COVID-19 and these other disasters, only to be blindsided by a “small scale disaster”. Now…to further complicate our Fire Safety Motivation attempts… over the past few years, data has emerged showing that the numbers of fires in the US are down from previous years. And that is correct, but I want to show you the actual numbers,….and tell you, the rest of the story. |
| 11 | This table is based on data reported to the NFPA (National Fire Protection Association) by US Fire departments over a 35 year period. It shows the numbers of fires in the US by type of fire and year. On this table, we are going to take a look at the first and second columns. |
| 12 | These two columns show the total numbers of fires corresponding to the years of 1980 through 2015. As I mentioned earlier, numbers of fire-related disasters have, for a number of reasons, decreased in the US in recent years. |
| 13 | This table starts in 1980 with a total of almost 3 million fires, for that year. |
| 14 | Down to a little over 1.3 million in 2015. We can see from this chart that fires are down, but it also shows us that just like any other disaster we can’t completely eliminate our threat from that disaster. And just like any other disaster, if can’t eliminate it, our top priority becomes Surviving it…or Life Safety. You will notice that this table did not address fatalities resulting from the fires. |
| 15 | According to the NFPA, home structure fires account for 92% of civilian structure fire deaths with the majority occurring between the hours of 11pm and 7am. In other words….when we are asleep and much less likely to be aware of the incipient phase of a fire. We can’t completely eliminate our risk from fire – so the key is being alerted to its existence in time to escape. |
| 16 | - Smoke Detectors are our alerting systems for homes. - Smoke Detector usage is credited as the greatest success story in the last part of the 20th century. - Smoke detector usage has increased from 10% in 1975 to 95% in 2000. - In addition, many of the fire safety improvements that have been made on larger occupancies have had a positive impact on small residential properties. It would appear that we have the problem satisfied in regards to being alerted in time to escape from a fire. However, fire death statistics tell a different story. |
| 17 | - These are stats complied by the US Fire Administration. These numbers agree with the previous numbers that we looked at by the NFPA. Just in a narrower, more recent time frame. |
| 18 | It shows numbers of fires decreasing 2.5% from 2009 to 2018. |
| 19 | This is the same chart, but showing the numbers for fire deaths during the same time period. As we can see, the numbers of fires deaths have increased 20.5%. So…..Something is not working. The following video will tell you: What you need to know. |
|  | INSTRUCTORS: Use one of the following videos: |
| 20 | ---MODUAL 1--- GMA/NEOFPA VIDEO (3:39.19 minutes) |
| 21 | or ---MODULE 1--- ROSSEN REPORTS VIDEO (6:20.01minutes) |
| 22 | Transition – Read Slide, One other thing that we want to mention: Dual Sensor alarms (Also called Combination alarms) may seem like the ideal solution to incorporate both types of technology, however as you may have noticed from the video, (Instructor: If only using the Rossen Reports video…note that this video does not test Dual Sensor Alarms) this type of detector tends to react slower to flaming fires than Ionization Detectors and slower to smoldering fires than Photoelectric detectors. Also, as to definitions, a Flaming Fire could also be referred to as a Blazing Fire, a Fast-Burning fire, a complete combustion fire, or a number of other names. With only a few exceptions, the Flaming fire can be defined by the presence of visible flame whereas a smoldering fire will show virtually all smoke, which is indicative of incomplete combustion. Now back to our key points: |
| 23 | Transition – Read Slide. With this last point in mind, let’s go back to a previous slide. |
| 24 | Transition –According to the NFPA… |
| 25 | Transition – Read Slide So……what this all means is that |
| 26 | Transition –Read Slide This next video is going to go over some Smoke Detector Basics. |
| 27 | ---MODUAL 2--- Smoke Detector Basics VIDEO (6:19.39 min.) INSTRUCTORS: If using video, after playing, skip to slide #41 (PowerPoint & Script). If not using video, continue with next slide (#28 PowerPoint & Script). |
| Or 28 | SMOKE DETECTOR BASICS SCRIPT The following Smoke Detector Basics may not cover all situations specific to your house and family. When you buy a detector, (hold up detector) you will find something like this in the packaging. (Take out & hold up guide) Usually called the Manufacturer’s User Manual or the Smoke Alarm User guide, or Smoke Detector Manual, or maybe just the User’s Manual. These Guides should be included in all detector packaging. They give you the complete information pertaining to that specific smoke detector. They also cover a lot of information that would apply to Smoke Detectors in general. I have two guides here (hold up two guides) these are from two of the major manufacturers. One is for a Photoelectric detector; one is for an Ionization detector. Both of these guides reference NFPA standards. Both of these user guides recommend that both Photoelectric and Ionization Detectors be installed for maximum protection. Some of the other information provided in these: Operation and Testing, Maintenance, Responding to an alarm, Using the different features of the alarm, Recommended locations, Nuisance alarms. They even have sections on Fire Safety in the home, such as developing and practicing a plan of escape. But the one section that’s probably the most important is “Limitations of Smoke Detectors”: Even though you may install the correct type of Smoke detectors, in the proper locations, the effectiveness of the detectors could be negated by other factors: a change in your situation, such as a door left open...or in some cases maybe closed, which could delay the smoke from reaching the detector, or channel it a different route, resulting in inadequate warning to the occupants, or even though the smoke reached the detector it may be difficult for the occupant to hear the alarm. Also, the condition of occupant, too young, too old, impaired mentally or physically can lead to their inability to escape, even though the detector had provided adequate time to escape. One of the manuals states that “Smoke Alarms are not foolproof”. They have to be selected, installed and maintained correctly for your specific needs and conditions, to minimize the possibility of failure. As we go through these basics, don’t worry about remembering everything, but be sure to refer to the user manuals. (Hold up guides) |
| 29 | In this module, we will go over… Terminology, Power types, Sensor types, Installation locations, Testing and Maintenance, and some additional tips. |
| 30 | - Terminology, for this program, Smoke Alarm and Smoke Detector - have the same meaning - Dual Sensor Alarm, Dual Sensor Detector, Combination Alarm, and Combination Detector - all have the same meaning. (Just indicates that they contain both types of sensor: Ionization and Photoelectric.) Don’t get confused with Combination alarms that may have two separate components: a smoke detector and a carbon monoxide detector or units that have these two separate components combined into the same unit. |
| 31 | Read Slide… Battery powered detectors, (either the 9 volt battery type or the Long-Life-Battery type) can be installed by the homeowner. Hardwired detectors should be installed by an Electrician. (We are talking about initial installation). |
| 32 | Read Slide |
| 33 | Read Slide |
| 34 | Read Slide UL: stands for Underwriters Laboratories: They do Product Safety Testing and certification. FM: stands for Factory Mutual: Insurer that develops Standards for the potential performance of building materials. |
| 35 | Read Slide This is a minimum. A smoke detector in every room reduces the notification time, increases the escape time and reduces the size of the fire firefighters will encounter. |
| 36 | Read Slide |
| 37 | Read Slide Some States have passed legislation requiring all smoke detectors to have “long-life” batteries. |
| 38 | Read Slide |
| 39 | Read first Slide bullet pt. For example: If your smoke detector alarms in the basement…you may not hear it on the second floor. Read second bullet pt. |
| 40 | Read Slide ---END VIDEO--- |
| 41 | Transition – (NEOFPA background screen) Now that we see the problem through the GMA Video and understand the basics of smoke detectors, we will now go over how to solve this problem. The next Video (or Module) will go over shopping for smoke detectors. |

| 42 | ---MODUAL 3--- Shopping for a Smoke Detector VIDEO (6:48.75 minutes) INSTRUCTOR: If using video, after playing, skip to slide #76 (PowerPoint & Script). If not using video, continue with next slide (#43 PowerPoint & Script). |
| --- | --- |
| Or43 | SHOPPING FOR A SMOKE DETECTOR SCRIPT When you go to shop for smoke detectors, it can be somewhat confusing. (Also be aware that you can also order Smoke Detectors on-line). |
| 44 | This is what you going to see in most stores that carry smoke detectors. We will go through some different types of detectors, and help you choose the ones that will keep you and your family the safest. Keep in mind that detectors come with many different features, different combinations of features, and different packaging, but remember you are looking for “Photoelectric Technology”. In the stores you will find many different types of smoke detectors: Some are powered by: - 9 volt batteries. With 10 year batteries. - Duel-Sensor sensor technology. (Combination Photoelectric and ionization.) - Hardwired detectors. - Detectors with voice….some with the mother’s voice recorded. - Combination Smoke and Carbon Monoxide Detectors - Test / Silence / Hush / and Mute Buttons - Detectors with Escape lights or exit lights - Interconnected Smoke Detectors (Hardwired or wireless) Choose the features that best fit your situation but remember…. The most important thing is to determine if the detector is “Photoelectric”. Looking for the letter “P” or word “Photoelectric “on the package. |
| 45 | These are some of the symbols that you may see on the packaging and/or back of the detectors. The far left column indicates the detectors with photoelectric technology….this is what we are looking for. The middle column shows the Ionization sensor symbol. The far right shows the Dual-Sensor symbols. These are not the only versions of symbols you may see, and they can be very small and difficult to find on the packaging. There also is no standardization as to their location on the product and packaging. |
| 46 | Also, remember to look for the UL (Underwriters Laboratories) or FM: (for Factory Mutual). Keep in mind that these labels may look slightly different on some products. So….let’s look at some Smoke Detectors… |
| 47 | This is our first detector. It is a 9 volt Battery operated Dual-Sensor Smoke Alarm…..you can see at the bottom left corner that it has both Photoelectric and Ionization Technology, in addition, it is UL Listed. For these slides we will use the red arrow for type of technology (…the sensor type) and the blue arrow for Listing type: UL or FM. |
| 48 | Therefore; on this detector, due to the Ionization technology, we do not recommend it. |
| 49 | This Detector has a Longlife battery, and it is UL listed, however; it has an “I” for ionization at the bottom left corner. |
| 50 | So we do not recommend it. |
| 51 | This detector is a hardwired detector that is wired into the house’s electrical system with a 9 volt battery backup. It is Ionization detector as you can see by the red arrow. It is UL listed. |
| 52 | So…Not recommended |
| 53 | This is a combination Smoke and Carbon Monoxide Alarm with a voice alarm. We can see that it is UL listed. We cannot see a symbol as to sensor type. It is in combination with a Carbon Monoxide detector but this does not necessarily indicate the type of technology the smoke detector has. |
| 54 | We would need more research on this one. |
| 55 | This is a Combination smoke and carbon monoxide alarm featuring two separate units. Both are powered by 9 volt batteries. It has an Ionization sensor in the smoke detector. It is UL listed. |
| 56 | This is not recommended due to the Ionization detector. |
| 57 | This alarm is a little different. We can see that it is UL listed. We can also see the Low price of $3.94. |
| 58 | We have to look a little farther on this one. |
| 59 | It’s hard to tell from this side of the box as to sensor type of alarm, let’s look further…. |
| 60 | This side tells us, in script, that it is an ionization detector. Note also: it says that it contains 0.9 microcurie of Americium 241 (Ammer-a- sea-um), a radioactive material that is produced by bombarding plutonium with high-energy neutrons). (Not that this is a health hazard…your watch will also show a reading on a Geiger counter). If you can’t determine the type of sensor on a detector, anytime you see that it contains a radioactive material, it indicates that it contains ionization technology. Also, the very low price, is usually an indicator that the detector is of the Ionization type. |
| 61 | Note this side of the package, The manufacturer strongly recommends that both ionization and photoelectric detectors be installed to help insure maximum detection of various types of fires that can occur within the home. If you read the small print on the packaging, many manufacturers’ are now recommending both technologies for maximum safety. |
| 62 | Due to the Ionization Technology, This type of detector is not recommended. |
| 63 | This is the card that a detector was attached to. It is 9 volt battery powered. It is UL listed. It says Photoelectric Sensor at the top. And also has the required Photoelectric symbol as indicated by the red arrow. |
| 64 | This detector would be recommended….assuming that a detector was attached to it. |
| 65 | Here’s a different kind of packaging: this is the back of the package, This detector is UL listed. It has Ionization technology. It has a 10- year lithium power cell…which is another way of referring to a Long Life Battery. |
| 66 | This detector would not be recommended due to Ionization technology. |
| 67 | This is the back of a detector, This is what it would look like if you take down one of your existing detectors in your house. This one is UL listed and has Ionization technology. |
| 68 | This detector would not be recommended. |
| 69 | This is the back of a different detector. You can see that it is Photoelectric. We cannot see a listing from this angle, but that may the bottom of the UL listing circle we see at the top of the picture. |
| 70 | With the information we have, it looks safe to say that this would be a recommended detector. |
| 71 | This is a box containing two 10 year sealed battery Smoke Alarms with intelligent wire-free interconnect. |
| 72 | This is the same box opened up. The instructions state that when one wireless alarm sounds, all wireless, interconnected alarms will sound. This gives the occupants more time to escape. These detectors can also send a notification to your smart phone. |
| 73 | This is the back of the box…it just explains some of the other features. |
| 74 | We can see on the end of the box that the detectors are Photoelectric and they are also UL listed. As of the taping of this program the cost of this combination is $79.00 |
| 75 | This detector would be recommended. ---END VIDEO--- |
| 76 | Read Slide In addition: Don’t think that just because cooking sets off an alarm that a fire necessarily will. |
| 77 | Read Slide |
| 78 | ---CONCLUSION--- So….How can I protect myself and my family from fire at home? - Buy Photoelectric Detectors. (Look for the letter “P” on the packaging or the back of the detector). - Add Photoelectric Detectors to your existing Detectors. - Install and maintain your Photoelectric detectors according to manufacturer’s recommendations. - React appropriately when the smoke detector alarms. |
| 79 | Slide only |
| 80 | Slide only |
| 81 | Slide only |
| 82 | QUESTIONS? |